| STRUCTURAL SPECIFICATION {{street}} |  |
| --- | --- |
| {{job_number}}. {{date}} | ENGSOLUTION ltd A 7/211 Ferry Road, Waltham T 03 365 0921 E admin@engsolution.co.nz |

- GENERAL STRUCTURAL CONSTRUCTION

  - GENERAL

This section forms an integral part of the contract documentation and shall be read in conjunction with the Conditions of Contract, Preliminary and General provisions, and all other relevant specification sections. It establishes fundamental requirements for structural construction works, including both general provisions and specific item descriptions.

The Contractor shall achieve full compliance with all applicable regulations, including but not limited to the New Zealand Building Code in its entirety, the Health and Safety at Work Act 2015, and all referenced standards. All construction activities shall adhere to industry best practices for structural integrity and site safety.

Prior to commencing any work on site, the Contractor shall ensure all contractually required insurance policies are in force and shall maintain such coverage without interruption throughout the project duration. The Engineer may require extension of insurance coverage to include risks such as vibration-induced loss of support, with such requirements to be confirmed in writing.

In the event of any discrepancy or conflict between consultants' specifications, the Contractor shall immediately notify the Design Engineer in writing and shall not proceed with affected work until receiving formal written instructions. The Contractor shall bear all responsibility for work performed without such authorization.

  - PROPRIETARY ITEMS AND ALTERNATIVES

The Contract Documents may specify certain proprietary items. The Contractor may propose alternative materials or components for the Engineer's approval, provided such alternatives are fully detailed and submitted at the tender stage. The Contractor shall account for all consequential changes and costs resulting from approved alternatives, including impacts on other trades, without entitlement to additional compensation from the Principal.

Approval of alternatives shall be granted only when the proposed equipment maintains equivalent dimensions and connection configurations to ensure proper fit within designated spaces, and when installation and maintenance access requirements remain uncompromised. All substitute items shall demonstrate equal or superior quality, finish, and performance characteristics compared to the specified products. The Contractor shall explicitly quantify and document any cost savings achieved through alternative selections, which shall be passed on to the Owner.

Unless expressly permitted in the Contract Documents, the Contractor shall provide the exact materials and components as specified. Strictly no substitutions shall be permitted without the Engineer's prior written authorization, which may require submission of technical data, samples, or test reports for evaluation. The Contractor shall allow reasonable time for the Engineer's review of any proposed alternatives and shall not proceed with substituted items until receiving formal written approval.

  - BUILDING ACT COMPLIANCE

    - CONTENT

The Contractor shall be responsible for obtaining all necessary consents and approvals required for the Works, including payment of all associated fees and charges levied by relevant authorities. Prior to commencing construction, the Contractor shall provide the Engineer with copies of all building consent conditions and any amendments to approved consent documentation.

In the event the Contractor elects to initiate site establishment or any construction activities prior to issuance of the building consent, written approval must first be obtained from both the Building Consent Authority and the Principal. The Contractor acknowledges that all such work undertaken without an issued consent shall be at their sole risk, unless otherwise expressly agreed in writing by the Principal.

MATERIALS & WORKMANSHIP

All materials and workmanship shall comply fully with the New Zealand Building Code (NZBC), irrespective of any omissions or inconsistencies in these specifications, drawings, or related documents. Proprietary items shall be designed, supplied, and installed strictly in accordance with the manufacturer’s published technical recommendations.

Materials, building elements, and construction methods shall satisfy the NZBC durability requirements for the specified 50-year building design life. The Contractor shall verify that all proposed materials meet the relevant NZBC clauses for:

- Structural performance (Clause B1)

- Durability (Clause B2)

- Protection from fire (Clause C)

- Other applicable performance criteria

  - HEALTH & SAFETY

The Contractor shall comply fully with:
(a) The Health and Safety at Work Act 2015 and all subsequent amendments;
(b) All relevant Acts, regulations, Codes of Practice, standards and guidelines issued by WorkSafe New Zealand; and
(c) The New Zealand Building Code provisions for construction safety, including but not limited to:
- F1: Hazardous Agents on Site
- F2: Hazardous Building Materials
- F4: Safety from Falling
- F5: Construction and Demolition Hazards

The Contractor shall implement and maintain effective health and safety management for all personnel:

- On the construction site; and

- In adjacent areas where safety may be affected by construction activities

Specific Requirements

- Training & Equipment
The Contractor shall ensure:
(i) All workers and subcontractors receive appropriate health and safety training;
(ii) Suitable safety equipment is provided and properly used;
(iii) Adequate precautions protect all persons entering the work area

- Site Facilities
(i) Maintain a stocked first aid kit on site and in all site vehicles
(ii) Prohibit smoking on site at all times during construction

- Neighbourhood Protection
The Contractor shall minimize nuisance from:

  - Dust generation

  - Noise pollution

  - Debris accumulation

  - Mud transfer to adjacent roadways or properties

- Work Methods
(i) Explosives are strictly prohibited for all project work
(ii) All demolition/construction methods must receive prior Engineer approval

- Reporting
Submit monthly health and safety reports to the Principal, including:

  - Incident logs (if any)

  - Hazard identification records

  - Corrective actions taken

Additional safety requirements are specified in the Preliminary and General section of this contract.

  - CONTAMINATION

No prior investigation or assessment of potential contamination hazards has been conducted for this site. The Contractor acknowledges that hazardous materials may be present, including but not limited to:

- Asbestos-containing materials (ACM)

- Biological contaminants (mould, rodent/bird droppings)

- Chemical residues

- Other unknown contaminants

The Contractor shall:

- Conduct all necessary assessments to identify potential contamination hazards prior to commencing works

- Engage qualified and experienced specialists as required for proper hazard identification and management

- Implement appropriate control measures in compliance with WorkSafe New Zealand regulations

- Include all provisional sums for contamination assessment and remediation as separately identifiable items in the Tender submission

  - SITE REQUIREMENTS

    - SETOUT & DIMENSIONS

The Contractor shall:
(a) Accurately establish site boundaries and set out all works in correct relation to these boundaries;
(b) Verify all structural dimensions and existing levels (including invert levels) prior to construction;
(c) Resolve any discrepancies between contract documents by consulting the Engineer and Architect; and
(d) Maintain all survey control points throughout construction.

Reference documents shall include the Architect's drawings for all construction dimensions.

SITE GANTRIES, FENCES & HOARDINGS

All temporary site enclosures shall comply with NZBC Clause F5 "Construction and Demolition Hazards" and shall prevent:

- Falling objects endangering persons or adjacent property

- Unauthorized site access (particularly by children)

- Off-site impacts from construction hazards

TEMPORARY SUPPORT

The Contractor shall:
(a) Design, construct, and maintain all necessary temporary works;
(b) Engage a professional engineer for specialist temporary works design and inspection;
(c) Coordinate with other contractors regarding support duration requirements; and
(d) Comply with all propping details shown on drawings.

Note: Construction method-related temporary works not shown on drawings remain the Contractor's responsibility.

ACCESS TO SITE

The Principal and their representatives shall have reasonable access for inspection during normal working hours.

DUST & NOISE CONTROL

The Contractor shall implement measures to minimize nuisance from:

- Dust generation

- Noise pollution

- Mud/debris transfer to adjacent properties

EROSION & SEDIMENT CONTROL

The Contractor shall:
(a) Prepare an Erosion and Sediment Control Plan complying with:

- Environment Canterbury Guidelines (2007)

- Christchurch City Council requirements

(b) Implement all measures prior to earthworks; and
(c) Maintain controls until site stabilization.

  - QUALITY ASSURANCE

GENERAL

The Contractor shall:
(a) Establish a documented QA system;
(b) Verify all works against contract documents; and
(c) Seek Engineer clarification for any uncertainties.

NOMINATED REPRESENTATIVES

For each structural trade, the Contractor shall:
(a) Appoint a qualified QA representative;
(b) Complete staged checklists;
(c) Submit checklists to Engineer before concealment; and
(d) Maintain organized QA records on site.

ENGINEER'S INSPECTIONS

The Contractor shall:
(a) Provide 2 working days' notice for inspections;
(b) Submit relevant QA documentation prior to inspection;
(c) Coordinate with Territorial Authority inspections; and
(d) Rectify non-compliant work with photographic evidence.

  - SHOP DRAWINGS

The Contractor shall prepare comprehensive shop drawings at their expense, detailing all components shown on the contract drawings along with complete construction information. These drawings must be developed in accordance with the trade-specific requirements outlined elsewhere in this specification. The Engineer's drawings provide principal dimensions, element sizes and typical details only, and the Contractor shall apply these typical details to non-standard locations using their professional expertise. Where uncertainties exist, the Contractor shall seek clarification from the Engineer.

All structural drawings shall be coordinated with architectural, services and other trade documentation. The Contractor shall verify all tolerances and clearances between elements, notifying the Engineer of any instances where adjustments are required to facilitate construction. Prior to submission, the Contractor shall conduct a thorough internal review of all shop drawings, evidenced either by completion of a submission form or company stamp.

Shop drawings shall be submitted to the Engineer for review no less than 14 days before planned manufacture. Any deviations from contract drawings shall be clearly identified and specifically brought to the Engineer's attention. The Engineer's review will focus on design concept and general arrangement compliance only, with dimensional accuracy remaining the Contractor's responsibility. Written approval from the Engineer must be obtained before commencing any manufacturing work.

  - DESIGN-BUILD COMPONENTS

This clause identifies the primary design-build elements of the contract, with full requirements detailed in relevant specification sections. The Contractor shall develop complete design solutions for the erosion and sediment control plan, concrete mix design meeting specified performance criteria, and temporary stability systems for precast panels and structural steelwork. All design-build components require submission of complete documentation including calculations, methodologies and certification.

  - QUALITY ASSURANCE CERTIFICATE (PS3)

Upon practical completion, when applying for the Code Compliance Certificate, the main Contractor shall provide a Producer Statement - Construction (PS3) encompassing all construction work including subcontractor elements. Additionally, trade-specific PS3 documents shall be submitted from each nominated representative, covering all relevant specification sections and incorporating all consultant advice notices and site instructions. The issuance and acceptance of these Producer Statements does not modify the Contractor's obligations regarding final completion, defects liability or maintenance period requirements.

- EXCAVATION AND HARDFILL

  - PRELIMINARY

This section shall be read in conjunction with the Preliminary and General Clauses and General Conditions of Contract, which apply equally to all trades. The requirements herein complement all other specification sections.

  - SCOPE

The Contractor shall provide all labour, materials, and equipment necessary for complete excavation and filling operations to establish structurally adequate foundations. This includes but is not limited to site preparation, bulk excavation, foundation excavation, hardfill placement, compaction, and backfilling.

  - RELATED DOCUMENTS

The Contractor shall comply with the latest versions of:

- New Zealand Building Code (BIA)

- NZS 4402:1986 Methods of Testing Soils (SANZ)

- NZS 4404:1981 Urban Land Subdivision (SANZ)

- NZS 4431:1989 Earth Fill for Residential Development (SANZ)

- Relevant concrete section of this specification.

  - QUALITY ASSURANCE

GENERAL

The Contractor shall ensure all work complies with the plans and specifications. Quality assurance procedures shall cover but not limited to:
(1) Set out of excavations
(2) Level of excavations
(3) Compaction of hardfill
(4) Backfill compaction
(5) Testing of subgrade
(6) Location of existing services
(7) Care and protection of existing buildings

INSPECTION

No backfilling, site concrete or reinforcing shall be placed without Engineer’s inspection. Minimum of 48 hours' notice shall be given where inspection is needed. Inspection does not relieve the Contractor of responsibility for specification compliance.

TESTING

An independent soils laboratory shall be engaged by the contractor to:
(i) Test existing subgrade after topsoil stripping using 3 passes of a 3-tonne vibrating roller, replacing soft spots with compacted sub-base metal
(ii) Verify compacted hardfill meets 4% CBR minimum (1 test per 100m²) and {{capacity}}kPa ultimate bearing capacity before building works commence

  - NATURE OF THE SITE

GENERAL

The Contractor shall refer to the provided full geotechnical report. All foundation design and details have been shown and included on the relevant structural drawings. The Contractor is deemed familiar with local conditions and shall make appropriate allowances.

LEVELS

All work shall maintain levels shown on Architects' and Engineers' drawings.

EXISTING SERVICES

The Contractor shall:
(1) Notify and pay fees to utility authorities, e.g. Gas and Electricity Authorities, the Local Council and Territorial Authorities, etc. 
(2) Locate, protect and repair existing services.
(3) Protect and restore footpaths/gutters, etc.
(4) Keep all thoroughfares clear

DUST AND NOISE

Implement measures to minimize nuisance to adjoining properties and the public.

PROTECTION OF ADJOINING PROPERTY

The Contractor shall take all precautions to prevent damage to adjoining property, with full responsibility for any restoration required.

EXPLOSIVES

Explosives use is prohibited.

VEGETATION REMOVAL

The Contractor shall remove all vegetation, stumps (including roots >25mm diameter) and obstructions form the site, disposing completely off-site.

TOPSOIL STRIPPING

The Contractor shall:

- Strip and removed all rubbish, demolition material, top soil and organic material from all arears subject to earthworks from the construction site.

- Remove all turf and organic topsoil prior to commencing other operations in these areas.

- Strip to sufficient depth to remove all organic material, turf and significant plant roots.

- Extend stripping 2m beyond earthwork limits except constrained by site boundaries, existing works or other physical limitations.

- Cooperate with the Engineer to determine appropriate stripping depths and avoid unnecessary over-excavation.

Where additional excavation depth is required by the Engineer, the additional excavation shall be agreed as a contract variation and obtain written instruction from engineer prior to commencing any backfilling or concreting operations.

  - EXCAVATION WORKS

EXCAVATION METHODOLOGY

The Contractor shall:

(1) Assume full responsibility for:

- Selection of excavation methods

- Stability of excavation faces

- And all associated risks on the selected methodology

(2) Execute all excavations:

- In a safe manner

- Without causing destabilization of site boundaries

- Without damaging neighbouring properties

All excavation shall be undertaken to industry practice standards

BULK EXCAVATION REQUIREMENTS

The Contractor shall perform bulk excavation to:

(a) Variable depths according to existing surface conditions

(b) Minimum depths sufficient to allow for:

- Placement of specified hardfill thicknesses

- 25mm blinding beneath slabs and foundations

(c) Achieve clean, undisturbed subgrade throughout

(d) Proof roll exposed surfaces using:

- Suitable compaction equipment

- Achieving 95% standard compaction (NZS 4431 Clause 7.4.1.1)

(e) Remove all surplus excavated material from site

FOUNDATION EXCAVATION

The Contractor shall:

(1) Excavate foundations to:

- The depths shown on drawings; or

- Competent bearing strata; whichever is deeper

(2) For hardfill-supported foundations, excavate to width equal to foundation width plus ½ hardfill depth each side

(3) Trim excavation bases to smooth, even surfaces

(4) Batter sides of temporary excavations to stable slopes

(5) Not excavate within 1V:2H slope of existing structures without:

- Prior Engineer notification

- Installation of approved ground retention systems

EXCAVATION MAINTENANCE

The Contractor shall maintain all excavations:

(a) Free from: Slips/Erosion/Water infiltration/Fallen materials

(b) Using appropriate: Slope batters/Pile liners/Shoring systems/Sheet piling/Planking/Strutting/Dewatering pumps

(c)Provide all necessary temporary supports and remove them when no longer required

SOFT SPOT TREATMENT

The Contractor shall:

(a) Excavate all soft areas as directed by the Engineer

(b) Remove all unsuitable material from site

(c) Backfill with compacted hardfill meeting Section 2.7 requirements

SERVICES EXCAVATION

The Contractor shall:

(1) Excavate for all services including, Plumbing, Drainage, Electrical, Gas, Telecommunications as shown on services drawings

(2) Backfill service trenches with compacted hardfill at 150mm maximum layer thickness. Proper consolidation between layers

(3) Comply with drainage specification requirements

INTERIOR EXCAVATIONS

For excavations within existing buildings:

(a) Minimize disruption to existing structures

(b) Sawcut slab edges cleanly:

- Extending to existing joints where possible

- Avoiding re-entrant corners

(c) Prevent undermining of existing elements using proper slope batters or temporary support systems

  - HARDFILL AND COMPACTION

Hardfill Material Specifications

All hardfill material shall:

(a) Comprise either basaltic rock or high-quality metal from approved sources

(b) Be well graded and compactable to specified densities

(c) Exclude materials that may weaken when wet

(d) Meet following gradation:

- Maximum particle size: 60mm

- 35-55% passing 19mm sieve

- ≤15% passing 600μm sieve

Placement and Compaction

The Contractor shall:

(1) Place hardfill in uniform layers across entire work area

(2) Compact with vibrating roller

(3) Maintain maximum uncompacted layer thickness of 150mm

(4) Conduct in-situ testing for density and moisture content at 10m x 10m grid intervals

(5) Achieve minimum compaction:

- 95% standard density (NZS 4431)

- 4% CBR (laboratory verified)

before proceeding with subsequent construction

Slab Subgrade Preparation

For areas under slabs-on-grade:

(a) Place minimum 300mm approved hardfill

(b) Compact to within 25mm of slab underside

(c) Finish with 25mm no-fines granular layer for DPM installation

(d) Verify final surface achieves 4% CBR minimum and 300kPa ultimate bearing capacity

Backfilling Requirements

The Contractor shall:

(a) Use approved backfill materials suitable for specific conditions

(b) Compact to specifications in Section 2.7.1/2.7.2

(c) Remove all debris prior to backfilling including rubbish, timber and other foreign matter

  - TRADE COORDINATION

The Contractor shall coordinate all related trades to ensure:

(a) Correct dimensional relationships between excavation works and other building elements

(b) Proper interface with drainlayers, plumbers for installation of drains/sumps/pipes/waste systems.

- CONCRETE - GENERAL

  - PRELIMINARY

The Contractor shall comply with:

(a) The Preliminary and General Clauses of this Specification; and

(b) The General Conditions of Contract

which provisions apply equally to all trades. This section shall be read in conjunction with all other specification sections.

  - SCOPE OF WORKS

This section covers:

- Formwork construction, concrete supply and placement

- Damp proof course (DPC) installation

- Precast component erection

- Reinforcing bar grouting

- Embedding of all bolts, plates and fixings

- Construction of foundations, walls, beams and in-situ concrete elements

Note: Reinforcing steel supply and installation is specified separately in the REINFORCING STEELWORK section.

  - REFERENCED DOCUMENTS

The following standards (latest editions) apply:

New Zealand Building Code (BIA)

NZS 3104 Specification for Concrete Production - High Grade and Special Grade.

NZS 3109 Specification for Concrete Construction.  (SANZ)

NZS 3112 Methods of test for Concrete.  (SANZ)

NZS 3114 Specification for Concrete Surface Finishes.  (SANZ)

Recommended Practice for Erection of Precast Concrete (PCI)

Alkali Aggregate Reaction (C&CA)

  - QUALITY ASSURANCE

General Requirements

The Contractor shall ensure all concrete works comply fully with:

- Contract drawings

- This specification

- Referenced standards

Inspections

The Engineer will inspect per NZS 3109 Clause 1.3.

Contractor must notify Engineer before pouring to inspect formwork/reinforcement/construction joints

All Engineer's instructions must be implemented before concrete placement

Concrete Testing

The Contractor shall conduct comprehensive concrete testing under the Engineer's supervision, strictly adhering to NZS 3109:1997 and NZS 3112:1986 standards. All testing shall be performed by an independent testing authority approved in writing by the Engineer.

Testing Frequency and Sampling

- Standard Testing Regime:

- For concrete pours less than 50m³ per day:

- One slump test

- One set of three (3) compression test cylinders

- For each additional 50m³ increment within a single day's pour, one additional set of three (3) compression test cylinders

- Special Circumstances Testing. Additional tests required for determining formwork stripping times shall be:

- Conducted separately from standard tests

- Funded entirely by the Contractor

- Performed using identical sampling methods

Testing Methodology

- Sample Collection:

- All test cylinders shall be taken by the Contractor's Foreman

- Strict compliance with NZS 3112:1986, Part 2 sampling procedures

- Immediate delivery of test specimens to approved laboratory (next day maximum)

- Test Analysis:

- The Engineer shall perform statistical analysis of results using:

- NZS 3109:1997 evaluation methods

- NZS 3112:1986, Part 2 protocols

- Full test reports shall be submitted to the Engineer within 7 days of testing

Non-Standard Testing Provisions

- Core Testing of Hardened Concrete:

- The Engineer may require core sampling when:

- Concrete quality appears questionable

- Proper pour notification wasn't provided

- Standard test results indicate non-compliance

- Cost responsibility. Contractor bears costs if: required due to lack of proper notification, OR Test results fail to meet specifications. Employer bears costs for all other core testing

- Additional Testing Authority:

- The Engineer reserves the right to require:

- Supplemental compression testing

- Tension testing

- Any other concrete performance tests

- At any project stage, regardless of standard testing schedule

- With immediate compliance required by Contractor

- With Engineer-provided testing specifications when applicable

Documentation and Reporting

- Test Record Requirements:

- Complete batch records for all concrete supplied

- Certified test results from independent laboratory

- Statistical analysis reports

- Core test results (when applicable)

- Non-compliance remediation plans (if required)

- Reporting Timeline:

- Preliminary slump test results: Within 4 hours of testing

- Compression test results: Within 7 days of specimen collection

- Core test results: Within 14 days of sampling

Non-Compliance Management

- Remedial Actions. Any concrete failing to meet specified strengths shall be:

- Reported immediately to the Engineer

- Evaluated for structural adequacy

- Remediated at Contractor's expense if replacement required

- Dispute Resolution. Testing disputes shall be resolved through:

- Independent third-party testing

- At Contractor's expense

- Using Engineer-approved laboratories

  - MATERIALS & WORKMANSHIP

General Compliance

All works shall comply with NZS 3109 unless specified otherwise herein. The Contractor shall maintain a copy of NZS 3109 on site.

Concrete Specifications

General Requirements:

- Special or High Grade concrete per NZS 3109 Clause 6.2

- Supplied by RMCNZ member plants (or Engineer-approved alternatives)

- Maximum aggregate size: 19mm (unless drawings specify otherwise)

- Calcium chloride prohibited

- Alkali aggregate reaction mitigation required

Mix Design Submissions:

- Submit for Engineer approval prior to construction

- Include evidence of alkali reactivity compliance

- No additives without written Engineer approval

- Plasticizers/superplasticizers prohibited without authorization

Strength Requirements:

The required 28-Day Compressive Strength for this project are:

Precast elements: {{precast_strength}} MPa

Foundations:	{{foundation_plain_strength}} MPa Plain Concrete / {{foundation_fibre_strength}} MPa Firbre Concrete

Metal deck topping: {{metal_deck_topping_strength}} MPa

Unless otherwise with written authorization from the design engineer, no additives/plasticizers/super plasticizers are to be included in the mix design.

Reinforcement

The Contractor shall comply with all requirements specified in the REINFORCING STEELWORK section. During concrete placement, the Contractor shall maintain continuous supervision to prevent reinforcement displacement and shall immediately rectify any displacement or damage at their own expense. A dedicated crew shall remain available throughout all pouring operations to address any reinforcement issues that may arise.

Structural Steelwork

All cast-in structural steel items specified in the STRUCTURAL STEELWORK section shall be properly positioned and secured prior to concrete placement. The Contractor shall verify all orientations and locations, and prevent any displacement during concrete operations. The Engineer shall inspect all weld plates before casting, and the Contractor shall ensure all contact surfaces are properly prepared by wire brushing or sandblasting.

Formwork

All formwork shall be constructed in strict accordance with NZS 3109 Section 5 requirements. The Contractor shall achieve precise dimensional control, producing straight lines and true surfaces that match the drawing dimensions exactly. Particular attention shall be given to ensuring all formed surfaces will meet the specified finish standards outlined elsewhere in this specification.

Construction Joints and Sawcuts

Construction joints shall be formed clean and dry with all laitance removed to expose aggregate, exactly as detailed on the drawings. For slab on grade, sawcut joints shall be executed within 24 hours of pouring, following the approved joint pattern. The Contractor shall coordinate with the Engineer to finalize joint locations before pouring and shall specifically avoid creating re-entrant corners, typically requiring diamond-shaped cuts around columns. All joints shall be properly sealed according to the manufacturer's specifications.

Floor Sealant

The Contractor shall apply Nitoseal SL floor joint sealant no earlier than 90 days after concrete placement. The sealing operation shall account for joint width increases due to shrinkage, with a second sawcut ensuring a minimum 20mm width at time of sealing. Backer rods shall be installed and all sealant applications shall comply strictly with manufacturer's specifications. The Contractor shall verify compatibility between the sealant and any proposed floor coatings.

Drilled and Epoxied or Grouted Bars

Holes for epoxy-grouted bars shall be drilled 8mm larger than the bar diameter and thoroughly cleaned with compressed air. Anchorage depth shall be the greater of 12 bar diameters plus 50mm or the dimension shown on drawings. Unless otherwise specified, the Contractor shall use Hilti HY-200 epoxy system or Sika 212 grout. Any proposed alternative products require the Engineer's written approval prior to use.

Tolerances

All work shall conform to the dimensional tolerances specified in NZS 3109 and NZS 3114 unless specifically modified herein. Structural elements shall maintain an absolute positional tolerance of ±5mm throughout the building height. Precast elements shall meet tighter tolerances of ±3mm for beams/columns and -0/+5mm for floor seating. Where multiple units interact, their combined tolerance shall not exceed that of a single element. The Contractor may request adjusted casting tolerances with Engineer approval when standard allowances prove inadequate.

Concrete Finishes

All concrete surfaces shall achieve the finish standards specified in NZS 3114. In general, slab finishes shall comply with Part 3, NZS 3114; exposed aggregate finishes with Part 2, NZS3114; and formed finishes shall be to Part 1, NZS 3114.

Ground floor slabs/waffle foundation shall receive a U3 finish suitable for coatings, while foundations shall achieve F1 (unexposed) or F3 (exposed) standards. Precast panels require an F5X finish with zero abrupt deviations. The Contractor shall coordinate with the Architect for specific requirements on external slab finishes.

Site Welding

All welding operations shall comply with the STRUCTURAL STEELWORK specifications. The Contractor shall properly prepare all surfaces by wire brushing or sandblasting before welding. Completed welds exposed in finished areas shall be cleaned of all slag and splatter, ground smooth, and finished to match adjacent surfaces.

Vibrators

The Contractor shall consolidate all concrete using vibration equipment and techniques that fully comply with NZS 3109 Section 7.6 requirements. The vibration operations shall ensure complete compaction without segregation, using appropriate vibration frequency and duration for each concrete mix and placement condition.

Dampproof Membrane

A continuous damp proof membrane consisting of two layers of 250 micron polythene shall be installed over all prepared subgrades beneath slabs. All joints shall have minimum 300mm overlaps sealed with 50mm wide pressure-sensitive tape. The membrane shall be turned up at all vertical elements and carefully sealed around penetrations. The Contractor shall protect the membrane throughout construction and immediately repair any damage occurring before slab placement.

  - CONCRETING OPERATIONS

General

All concrete placement operations shall strictly adhere to the requirements of NZS 3109 Section 7. The Contractor shall employ proper placement techniques to prevent segregation and ensure complete consolidation of all concrete elements.

Curing and Protection

The Contractor shall cure all concrete in accordance with NZS 3109 Clause 7.8, using methods appropriate for the specific concrete mix and environmental conditions. When proposing to use curing compounds, the Contractor must obtain the Engineer's specific approval for each application, demonstrating compatibility with any subsequent finishes or coatings.

Rebates, Reveals and Architectural Details

All architectural concrete features including rebates, reveals, and special edges shall be formed exactly as shown on the architectural drawings. Particular care shall be taken forming openings for windows, doors, and service penetrations to ensure proper fit and finish of all interfacing components.

Building In

The Contractor shall properly install and secure all embedded items including service penetrations, sleeves, conduits, and cast-in fixtures required by other trades. All such items shall be accurately positioned and firmly anchored to prevent displacement during concrete placement operations.

- REINFORCING STEEL

  - PRELIMINARY

The Contractor shall comply with both the Preliminary and General Clauses of this Specification and the General Conditions of Contract, which apply equally to all trades. This section shall be read in conjunction with all other specification sections.

  - SCOPE

This section covers the supply, bending and placement of all reinforcement for concrete and reinforced blockwork, including distance pieces and spacers. While precast concrete and piling subcontractors may handle their respective reinforcing, all materials must comply with this specification. Blockwork reinforcing placement may be performed by the blockwork subcontractor, except for starter bars.

  - RELATED DOCUMENTS

The following standards (latest editions) apply:

- The New Zealand Building Code. (BIA)

- NZS 3101:1995 Concrete Structures Standard (SANZ)

- NZS 3109:1997 Specification for Concrete Construction  (SANZ)

- NZS 3402:1989 Steel Bars for the Reinforcement of Concrete.  (SANZ)

- NZS 3421:1975 Hard drawn mild steel wire for concrete reinforcement.  (SANZ)

- NZS 3422:1975 Welded fabric of drawn steel wire for concrete  (SANZ) reinforcement.

- NZS 4701:1981 Metal arc welding of Steel Structures.  (SANZ)

- NZS 4702:1982 Metal arc welding of grade 275 reinforcing bar. (SANZ)

- BS  5896:1980 High tensile steel wire and strand for the (BSI) prestressing of concrete.

- AS/NZS 4671:2001 Steel Reinforcing Materials (SANZ)

Refer also to Piling, Ground Anchors, and Structural Steelwork sections.

  - QUALITY ASSURANCE

General

The Contractor shall ensure all reinforcement complies fully with the drawings and this specification.

Inspection

The Engineer will inspect all reinforcement according to the drawing schedule, requiring at least 48 hours' notice before each inspection.

Testing

The Contractor shall provide manufacturer's certificates and certificates of origin for all reinforcement. If certificates indicate potential non-compliance, the Contractor shall fund additional testing required by the Engineer to verify compliance.

  - MATERIALS AND WORKMANSHIP

General

All materials and workmanship shall conform to NZS 3109 requirements.

Reinforcing Steel

Grade 300E bars (marked D or R) shall be NZ-manufactured complying with NZS 3402:1989. Grade 500E bars (marked H or HR) shall be NZ-manufactured Pacific Steel micro-alloyed bars complying with NZS 3402:1989. Quench and tempered bars are prohibited without written Engineer approval. Any non-compliant steel shall be removed at the Contractor's expense. The Engineer requires steel authentication certificates before any concrete pouring.

Alternative Reinforcing

Alternative reinforcing requires the design engineer's written approval.

Reinforcing Mesh

Suspended floor mesh shall have ≥10% uniform elongation (AS/NZS 4671). Slab-on-grade mesh shall be hard drawn ductile steel wire (NZS 3421), with finished fabric complying with NZS 3422.

Galvanizing

Galvanized bars shall be coated before bending with ≥700g/m² coating.

Bending of Bars

Bars shall be cut and bent per NZS 3109 Section 3 and drawing dimensions. Bend diameters for longitudinal reinforcing shall comply with Clause 5.5.7. Stirrups shall use standard NZS 3109 shapes, with larger diameters required around ducts and mechanical splices. Lapped cranks/sets shall have 12d or 300mm inclined portions unless shown otherwise.

Bend Diameters

Minimum bend diameters shall be:

| Steel Grade | Bar Size | Minimum diameter of bend diameter (mm) |  |
| --- | --- | --- | --- |
|  |  | Plain Bars | Deformed Bars |
| 300E and 500E | 6-20 | 2 bar diameters | 4 bar diameters |
|  | 24 | 3 bar diameters | 6 bar diameters |

For galvanised deformed bars, the minimum bend diameter shall be

- Bar size 16dia or less: 5 bar diameters

- Bar size 20dia or greater: 8 bar diameters.

Abbreviations

Drawing abbreviations include:

| D | deformed grade 300E bar |
| --- | --- |
| R | Plain round grade 300E bar |
| H | deformed grade 500E bar |
| HR | Plain round grade 500E bar |

Cover

Minimum cover shall be per drawings (generally exceeding NZS 3101). NZS 3109 tolerances apply, with zero tolerance for reduced cover. Tie ends shall face away from concrete surfaces. Where exposure varies, use maximum required cover.

Laps in Concrete

Lap lengths shall be per drawings or NZS 3101:2006 (minimum 300mm). Plain bars require twice the deformed bar lap length.

| 28-DAY CONCRETE STRENGTH | Grade 300E Minimum Lap Length (mm) |  |  |  | Grade 500E Minimum Lap Length (mm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D10 | D12 | D16 | D20 | HD10 | HD12 | HD16 | HD20 |
| 20MPa | 350 | 450 | 550 | 700 | 600 | 700 | 900 | 1150 |
| 25MPa | 300 | 400 | 500 | 600 | 500 | 600 | 800 | 1000 |

Laps in Blockwork

| Bar Diameter (mm) | Grade 300E Minimum Lap Length (mm) | Grade 500E Minimum Lap Length (mm) |
| --- | --- | --- |
| 10 | 400 | 700 |
| 12 | 500 | 850 |
| 16 | 650 | 1150 |
| 20 | 800 | 1400 |

The Contractor shall also comply with all requirements specified in the MASONRY BLOCK section.

Welding of Bars

All welding shall comply with AS/NZS 1554.3 and Structural Steelwork section. Welding 300E/500E bars requires Engineer approval and approved procedures. Generally, only NZS 3402/AS/NZS 4671 compliant bars may be welded.

Welding Inspection

Refer to Structural Steelwork section for inspection requirements.

Heating of Reinforcement

Heating 300E/500E bars requires Engineer approval and approved procedures.

Re-bending of Reinforcement

Re-bending 300E/500E bars requires Engineer approval and approved procedures.

- STRUCTURAL STEELWORK

  - PRELIMINARY

The Contractor shall comply with the Preliminary and General Clauses and General Conditions of Contract, which apply equally to all trades. No modifications to structural elements shall be made without written approval from the Design Engineer. This section shall be read in conjunction with all other specification sections.

  - SCOPE

This section covers the complete supply, fabrication, surface treatment, delivery and erection of all structural steel elements shown on the structural and architectural drawings. The scope includes all weld plates, bolts, cleats and related components for concrete and blockwork embedment, along with all necessary temporary works and access equipment. The Contractor shall provide all steel purlins, bracing systems, portals, and framing members complete with all connection components as detailed.

  - CONSTRUCTION CATEGORY

All steelwork shall comply with the following classifications unless noted otherwise:

- Importance Level: IL2

- Service Category: SC1

- Fabrication Category: FC1

- Construction Category: CC2

  - RELATED DOCUMENTS

The following standards (latest editions) apply:

- The New Zealand Building Code (MBIE)

- AS/NZS1554.1:2011

- NZS3404:1997 Design of Steel Structures and related documents – refer appendix A of the code

- AS/NZS 2312 Guide to the Protection of Structural Steel against Atmospheric corrosion by the use of protective coatings and related documents – refer section 1.4 of code.

- AS4600 Cold formed Steel Structures Code

  - WORKMANSHIP AND MATERIALS

General Requirements

All materials and workmanship shall comply with NZS 3404:1997 Code for Design of Steel Structures and AS 1554 Structural Steel Welding Code, both on and off-site.

Structural Steel Materials

Steel sections shall meet minimum yield strengths:

- PFC, flats, angles, UB/UC: 300 MPa

- CHS, RHS, SHS: 350 MPa

- Moment splice plates: 350 MPa

- Flats/plates: 300 MPa

The Contractor shall verify all steel properties through batching sheets and demonstrate full compliance for any substituted materials. Fabrication shall achieve precise fit-up with all similar components being interchangeable, performed in accordance with SAA MA 1.8 standards. Site measurements shall confirm all concrete dimensions before fabrication.

It’s the fabricator's responsibility to ensure all concrete and fixing dimensions on site prior to fabrication.

All alterations require engineer’s written approval prior to fabrication. Where that the contractor failed to achieve engineer’s written approval and the yield strengths/steelwork sections supplied do not match the design drawings, it’s the contractor’s responsibility to demonstrate that the supplied steelwork complies in full with this specification and its nominated reference. The engineer may request to replace the incorrect member. Contractor would be responsible for all associated cost.

Fabricated Sections

Fabricated members shall match specified section properties and dimensions, complying with NZS/AS 3679 and NZS 3404 fabrication tolerances. Full penetration double-sided welding is required unless otherwise approved by the Engineer.

Secondary Steel Elements

Girts and purlins shall be Grade 500 galvanized steel with pre-punched holes. All sag rods, bolts and bracing channels shall be galvanized and installed per manufacturer specifications and drawing requirements.

Bolting Requirements

Bolting shall comply with:

- General connections: Grade 8.8/S to AS 1252 unless otherwise noted in the drawings.

- Purlins/sag rods/timber connections: Grade 4.6/S to AS 1111:1980

All bolts and nuts shall be in accordance with following standars.

| 4.6/S bolts | to | NZS/AS 1252: 2016 |
| --- | --- | --- |
| 8.8/S bolts | to | NZS/AS 1252: 2016 |
| 8.8/TF bolts | to | NZS/AS 1252: 2016 |
| Nuts | to | NZS/AS 1252: 2016 |
| High Strength Nuts | to | NZS/AS 1252: 2016 |

All bolts shall be installed with proper thread engagement. Engineer’s approval would be required for the following locking systems: lock washers, self-locking nuts, or locknuts. Snug-tight bolts shall be tensioned per NZS 3404:1997.

Welding Standards

All welding shall comply with NZS/AS 1554 Part 1: 2014 Welding of Steel Structures, performed by welders certified to AS/NZS ISO 9606.1:2017 Qualification of welders for fusion welding of steels.

Standard welds shall be Category SP unless noted otherwise.

The Contractor shall provide proper staging, fire protection measures, and inspection access for all welding operations.

Specialist welding advice and inspections may be requested by the engineer to confirm his own inspections at the cost of the client.

The Contractor shall conduct all weld testing in strict accordance with AS 1554, Part 1:2014, including both:

- Standard test specimens; and

- Actual production welds with parent metal samples extracted from the work

The testing regime shall comply with the comprehensive requirements of AS 1554, Part 1:2014, incorporating:

- Visual inspection of 100% of welds

- Non-destructive testing per AS/NZS 5131:2016 Tables I6 and I7, based on:

  - Weld Failure Consequence Category

  - Seismic Demand Category

- Additional testing as specified in the following table:

| Structural Element | Weld Failure Consequence | Seismic Demand | Testing Requirements |
| --- | --- | --- | --- |
| Portal Frame Connections | Category A | Medium (M) | Enhanced NDT per Table I7 |
| All Other Steelwork | Category C | Low (L) | Standard NDT per Table I6 |

The Contractor shall:

- Submit complete test results to the Engineer for review and approval

- Obtain written acceptance before concealing any welds

- Maintain full traceability of all tested welds to their locations in the structure

Any welds failing to meet acceptance criteria shall be:

- Clearly marked and documented

- Repaired using approved procedures

- Retested to original requirements

- Reported to the Engineer with corrective action records

The Contractor shall employ a certified welding supervisor (AWS/CWI or equivalent qualification) to continuously monitor all welding operations. The supervisor shall verify welder qualifications, maintain daily inspection records, enforce strict compliance with AS/NZS 1554.1:2014 standards, and report directly to the Engineer on all welding quality matters.

All welding shall be performed under controlled shop conditions unless specifically approved in writing by the Engineer. Approved site welding shall utilize hydrogen-controlled electrodes (AWS A5.1 E7018 or equivalent) and comply with approved welding procedure specifications developed for field conditions, including proper environmental controls such as wind breaks and preheat requirements.

For connections to existing steelwork, the Contractor shall prepare surfaces to SSPC-SP10/NACE No. 2 near-white metal standard by complete removal of all coatings, rust and contaminants. Material testing shall be conducted to determine steel composition, weldability characteristics and required thermal controls. The Contractor shall develop and submit weld procedure specifications for Engineer approval, including trial welds on representative samples before commencing production welding.

All welding defects identified through inspection shall be documented with photographs and sketches before remediation. Defective areas shall be completely removed by grinding to sound metal and repaired using approved procedures including minimum 100mm overlap with sound weld metal, increased preheat temperatures (minimum +50°C above standard requirements), and post-weld heat treatment where specified. Repaired welds shall be retested to original requirements plus enhanced inspection criteria including 100% visual examination, 200% increase in original NDT volume, and macro-etch testing for critical connections.

The Contractor shall bear all costs associated with defect investigation, remedial welding, additional testing and work delays when defects result from improper procedures, unqualified personnel, non-compliant materials or inadequate supervision. The Engineer may require extended testing of adjacent welds in the same heat lot, similar connections throughout the structure, or random sampling of other production welds using methods specified in HERA Design Guide Volume 2 Section 17 Figure 7.2.3.2.

Erection Requirements

The erection sequence and temporary bracing shall be approved by the Engineer. All steelwork shall be erected without overstressing, maintaining true alignment. Temporary cleats shall be removed and finished smooth. Cambers and presets shall be fabricated as shown on drawings to achieve proper final geometry under self-weight.

Cambers or presets

The Contractor shall fabricate all structural steel members with the precise camber dimensions specified on the approved construction drawings. Where presets are indicated, the steelwork shall be fabricated to the designated preset dimensions to account for anticipated deflections, ensuring the structure achieves its intended neutral position under self-weight loading conditions.

The fabrication shall incorporate these geometric adjustments while maintaining all specified material properties and dimensional tolerances in accordance with NZS 3404:1997. The Contractor shall verify all camber and preset measurements during shop fabrication using certified surveying equipment, with verification records submitted to the Engineer for review prior to shipment.

For members requiring camber, the Contractor shall:

- Construct the specified vertical curvature during fabrication

- Maintain proper flange alignment throughout the cambered length

- Ensure all connection details accommodate the cambered geometry

- Clearly mark the orientation of each cambered member

For members requiring presets, the Contractor shall:

- Fabricate to the exact preset dimensions shown

- Account for all anticipated dead load deflections

- Coordinate preset requirements with erection sequencing

- Verify member geometry after release of temporary supports

All camber and preset measurements shall be verified both in the fabrication shop and after erection, with any deviations exceeding ±5mm or 5% of specified camber (whichever is smaller) requiring Engineer notification and corrective action. The Contractor shall provide certified dimensional control reports documenting as-fabricated and as-erected measurements.

Roof Bracing

Diagonal bracing systems shall be installed per manufacturer instructions and structural drawings, using specified Donobrace/Reidbrace components.

  - FABRICATION

General Fabrication

All fabrication shall comply with NZS 3404 Section 14.

The Contractor shall verify all critical dimensions through site measurements before fabrication. This includes checking structural interfaces, gridlines, foundation locations, and connection details against the drawings. Measurements must use calibrated equipment and account for actual site conditions.

The steelwork design shall accommodate potential field adjustments where pre-fabrication verification isn't possible. No formal report is required, but the Contractor remains responsible for all dimensional accuracy.

Cutting and Holing

The Contractor may employ shearing, sawing, or thermal cutting methods for steel fabrication unless otherwise specified. Thermal cutting shall be machine-operated wherever practicable, with manual methods restricted to section shapes, copes, repairs, and applications where machine cutting is unfeasible.

All cut surfaces shall be finished square and dimensionally accurate, free from excessive roughness or defects that could compromise structural performance or interfere with fabrication processes. Re-entrant corners shall be formed without notches, maintaining a minimum radius of 10mm unless detailed otherwise on the drawings.

Cut edges requiring weld preparation shall be ground smooth to remove all slag and oxidation prior to fabrication. The Contractor shall verify all critical cut dimensions using calibrated measuring equipment, ensuring compliance with specified tolerances before proceeding with assembly. Thermal-cut surfaces destined for paint systems shall be prepared to SSPC-SP6 commercial blast cleanliness standards.

The Contractor shall drill all holes for structural connections and trade fixings as specified on the approved drawings. Hole dimensions and spacing shall comply with the following minimum standards:

- Edge distances shall maintain at least 2D (where D = nominal bolt diameter) from bolt center to steel edge, unless shown otherwise on the structural drawings.

- Bolt spacing shall be no less than 2.5D between adjacent fasteners center-to-center, except where specifically detailed.

- Standard hole diameters shall be:

  - D + 2mm for bolts ≤24mm diameter

  - D + 3mm for bolts >24mm diameter

- Slotted holes shall match the dimensions specified on the structural drawings, with their longitudinal axis perpendicular to the direction of load transfer unless otherwise indicated.

All holes shall be cleanly drilled (not punched) with perpendicular faces and burr-free edges. The Contractor shall verify hole patterns using certified templates prior to assembly, ensuring proper alignment of mating components. Any field modifications to hole patterns require the Engineer's prior approval.

  - FINISHES

    - GENERNAL REQUIREDMENTS

The Contractor shall design, specify and apply all protective coating systems in compliance with NZS 3404 and AS/NZS 2312 standards. The complete coating system must provide adequate protection during transport, erection, and temporary exposure.

All protective coating systems shall fully comply with NZS 3404 and AS/NZS 2312 standards for material selection, application, and repair. The Contractor shall ensure the completed system meets both this specification and the referenced standards. Any conflicts between documents must be resolved with the Engineer prior to work commencing. The Contractor shall submit coating specifications for review, maintain quality records, and verify compliance before final acceptance. This applies to all coating types including paints, galvanizing, metal spraying, and intumescent coatings. Final approval requires demonstration of full compliance with all requirements.

    - SURFACE PREPARATION AND COATING APPLICATION

The Contractor shall prepare all steel surfaces to meet coating manufacturer specifications, ensuring proper adhesion and system longevity. All welds must be cleaned of slag and spatter, with exposed welds ground smooth. Surface preparation shall include complete removal of sharp edges, grease, oil, and contaminants as specified by the coating manufacturer.

For fire-protected steelwork, the Contractor shall use a fully compatible coating system including primer, intermediate coats, and intumescent topcoats. Contractor is responsible to ensure all necessary coatings are equivalent and equivalence with be insisted upon. FAILURE OF SUCH WILL RESULT IN COATING REMOVAL AND REPAINTING ON CONTRACTOR’S COST.

Steel to be embedded in concrete beyond 50mm shall be cleaned to bare metal by power tool and left unprimed.

THE PROPOSED PAIN SYSTEM MUST BE SUBMITED BY THE CONTRACTOR FOR ALL STEELWORK TO THE ENGINEER FOR COMMENT BEFORE PROCEEDING WITH ANY PAINT APPLICATION.

Surface preparation shall occur after fabrication completion, with coating application following within four hours. The Contractor shall apply shop coatings per manufacturer recommendations, limiting field painting to necessary joint areas only. All coating repairs and touch-ups shall maintain compatibility with existing systems, with adjacent areas protected during welding operations.

    - GALVANIZING AND METAL SPRAYING REQUIREMENTS

All exposed structural steelwork, including connection components such as bolts and plates, shall be hot-dip galvanized unless specifically exempted on the drawings. Galvanizing shall comply with the current edition of the Galvanizers Association of New Zealand Manual, meeting requirements for surface preparation, minimum coating thickness (typically 85μm), and finished quality. Where architectural or performance requirements dictate, galvanized surfaces shall receive compatible paint systems as specified in the Surface Finishes Schedule.

    - PAINT SYSTEM SPECIFICATION

The Contractor shall develop paint systems in accordance with NZS 3404:Part 1:2009, selecting products appropriate for each steelwork application based on environmental exposure and accessibility for maintenance. Proposed systems shall include complete technical specifications demonstrating compliance with the durability requirements outlined in the Surface Finishes Schedule. All paint systems must be submitted to the Engineer for approval prior to procurement or application, including full product data sheets and compatibility statements for multi-coat systems. The specification shall account for required surface preparation methods, application procedures, dry film thicknesses, and quality control measures for each exposure condition.

The Contractor shall implement protective coating systems meeting the following minimum performance standards based on exposure conditions:

Internal Steelwork Protection

- Dry Conditions (C1 Classification):

  - Environment: Interior spaces within insulation line

  - Minimum durability: 15 years to first maintenance

  - System: Primer + compatible finish coat(s)

- Damp Conditions (C3 Classification):

  - Environment: Enclosed spaces outside insulation line

  - Minimum durability: 25 years to first maintenance

  - System: Corrosion-inhibitive primer + moisture-resistant topcoat(s)

External Steelwork Protection

- Sheltered Conditions (C3 Classification):

  - Environment: Areas protected from direct rainfall

  - Minimum durability: 25 years to first maintenance

  - System: High-build protective coating system

- Exposed Conditions (C2 Classification):

  - Environment: Areas subject to regular wetting

  - Minimum durability: 15 years to first maintenance

  - System: Weather-resistant industrial coating system

- Wet Conditions (C4 Classification):

  - Environment: Continuously damp or immersed areas

  - Minimum durability: 25 years to first maintenance

  - System: Specialist corrosion protection system

THE PROPOSED PAIN SYSTEM MUST BE SUBMITED BY THE CONTRACTOR FOR ALL STEELWORK TO THE ENGINEER FOR COMMENT BEFORE PROCEEDING WITH ANY PAINT APPLICATION.

The Contractor shall comply with all architectural paint specifications in addition to these requirements. Where indicated on the drawings, intumescent fire protection coatings shall be applied to specified steelwork members. The selected intumescent system must demonstrate full compatibility with the approved primer system through manufacturer's certification and compatibility testing.

Surface preparation for intumescent coatings shall meet both the general requirements of Section 6.7.2 and the specific preparation standards stipulated by the coating manufacturer. The Contractor shall apply all protective coatings in strict compliance with the manufacturer's published application guidelines, including:

- Environmental conditions (temperature, humidity)

- Surface preparation standards

- Application methods and equipment

- Required dry film thicknesses

- Intercoat curing times

- Quality control procedures

    - ERECTION TOLERANCES

All steelwork erection shall comply with NZS 3404:Part 1:2009 tolerances unless otherwise specified. The Contractor shall verify and maintain dimensional compliance for all structural elements, connections, and interfaces. Non-conforming work shall be corrected at the Contractor's expense. Cumulative tolerances over the full building height shall be strictly controlled.

    - HEALTH AND SAFTY

The Contractor shall comply with all applicable health and safety regulations, including the New Zealand Building Code provisions for hazardous materials (F1/F2), fall protection (F4), and construction hazards (F5). Temporary bracing must maintain structural stability during erection, with the Contractor responsible for all temporary works design. The steel structure shall remain stable throughout subsequent construction phases.

  - FIRE PROTECTION

Intumescent Coating Requirements

All designated steelwork shall receive intumescent coatings achieving the specified Fire Resistance Rating (FRR). The Contractor shall use manufacturer-approved primer and topcoat systems, applying all materials strictly per published specifications. A written warranty shall confirm the installed system meets the required FRR through proper dry film thickness application.

Fire Rating Compliance

The fire protection system shall comply exactly with the structural fire rating drawings and fire engineer's specifications. The Contractor must obtain written certification from the fire engineer verifying all FRR requirements are met. All connection components (bolts, cleats, etc.) shall receive equivalent fire protection unless specifically exempted.

Finished Appearance

The final intumescent coating shall match the architect's specified finish requirements while maintaining full fire protection performance. The system must remain compatible with any architectural topcoats specified.

- STRUCTURAL TIMBER

  - SCOPE

This section covers all structural timber elements including:

- Solid sawn timber beams and framing members

- Glue-laminated (GL) beams

- Laminated veneer lumber (LVL)

- Structural plywood sheathing

  - MATERIAL REQUIREMENTS

Species and Grading

All structural timber shall be either:

- Radiata Pine (Pinus radiata); or

- Douglas Fir (Pseudotsuga menziesii)

manufactured to verified stress grades unless alternative species are approved in writing by the Engineer. The Contractor shall provide certified grading documentation for all timber materials.

Strength Requirements

Timber elements shall meet the following minimum strength grades:

| Component Type | Minimum Stress Grade |
| --- | --- |
| Solid sawn timber | SG8 |
| Glue-laminated beams | GL8 |
| LVL beams | LVL11/LVL13 |

All timber shall be:

- Kiln-dried to moisture content appropriate for end use

- Free from excessive defects affecting structural capacity

- Properly graded and marked per NZS 3631

- Protected from moisture during storage and installation

Timber treatment and moisture content Requirements

All structural timber shall be preservative-treated in accordance with NZS 3640:2003 to meet the durability requirements of B2/AS1. The following minimum treatment classes apply:

- H1.2 Treatment:

  - For timber fully enclosed within the building envelope

  - Not exposed to wet areas or moisture

- H3.2 Treatment:

  - For timber exposed to:

    - Internal wet areas (bathrooms, laundries)

    - External environments (not in ground contact)

- H5 Treatment:

  - For all timber in direct ground contact

  - For critical structural elements requiring maximum protection

Treatment stamps shall remain visible after installation for verification purposes.

Moisture Content Specifications are as follows:

- Framing Timber:

  - Acceptable range: 12-18% moisture content at installation

  - Kiln-drying required for all load-bearing wall components

  - Verification: Resistance-type moisture meter testing when requested by Engineer

- Glue-Laminated Timber:

  - Moisture content at time of gluing: 10-14%

  - Maximum variation between adjacent laminates: 2%

  - Factory certification required for moisture content compliance

- Protection During Construction:

  - All timber shall be protected from moisture absorption

  - Proper ventilation must be maintained in enclosed spaces

  - Wetting of structural timber elements is prohibited

The Contractor shall maintain records of all treatment certificates and moisture content testing for review by the Engineer. Any timber failing to meet these requirements shall be rejected and replaced at the Contractor's expense.

CONNECTION HARDWARE REQUIREMENTS

All structural connections including bolts, washers, plates and connectors shall be hot-dip galvanized for corrosion protection. Bolt washers shall comply with NZS 3603:1992 Clause 4.4 requirements, with the following minimum dimensions:

| Bolt Diameter | Washer Size (mm) | Thickness (mm) |
| --- | --- | --- |
| Up to M8 | 25 × 25 | 1.5 |
| Up to M12 | 50 × 50 | 3 |
| Up to M20 | 65 × 65 | 5 |

Alternative washer dimensions shown on structural drawings shall take precedence over these standard requirements.

TIMBER-TO-STEEL CONNECTIONS

- Timber Preparation

For steel plate connections in beam/column ends or rebates:

- Slot width shall not exceed plate thickness by more than 2mm

- Recess depth shall not exceed plate thickness by more than 1mm

- All cut surfaces shall be clean and square

- Tolerances shall be verified before plate installation

- Steel Connection Plates

All steel connection components shall:

- Comply with Section 6 (Structural Steelwork) requirements

- Be fabricated to drawing dimensions

- Have clean, square-cut edges

- Be free of burrs and sharp edges

- Be properly galvanized as specified in 7.2.4

The Contractor shall verify all connection fits prior to assembly and report any discrepancies to the Engineer. All connection hardware shall be installed in accordance with the manufacturer's specifications and NZS 3603 requirements.

- MASONRY BLOCKWORK

  - GENERAL REQUIREMENTS

All masonry blockwork shall conform to NZS 4210 and NZS 4230, with construction directly supervised by a registered NASIB supervisor providing continuous inspection. The Contractor shall ensure compliance with all relevant standards, specifications, and the New Zealand Building Code.

  - MATERIALS

Blockwork: All masonry blocks shall comply with AS/NZS 4455, with lightweight aggregate prohibited. Full-size units shall be used where possible, minimizing cut blocks.

Mortar: Mortar shall achieve a minimum 28-day compressive strength of 12.5 MPa and a 7-day masonry-to-mortar bond strength of 200 kPa, conforming to NZS 4210. Joints shall be uniform (nominally 10 mm thick), level, and plumb, laid in running bond pattern unless otherwise specified.

Grout: Grout shall have a minimum 28-day compressive strength of {{grout_strength}}, using coarse aggregate with a nominal size range of 4.75 mm to 13.2 mm. An approved expansive additive shall be included, and grouting shall be performed using the high-lift method in accordance with NZS 4210.

Reinforcement: All reinforcement shall be Grade 300E or 500E per AS/NZS 4671, as detailed on structural drawings. No substitutions in bar grade, size, or strength are permitted without prior written approval from the Engineer.

  - CONSTRUCTION REQUIREMENTS

Shrinkage Joints: Where not indicated on drawings, shrinkage joints shall be constructed at maximum 6.0 m centers, in accordance with NZS 4210 and to the Engineer’s approval. Joints shall feature continuous vertical perpends, with horizontal reinforcing bars and full-height grout continuity.

Placement & Inspection: No grout shall be placed until the Engineer has inspected and approved all blockwork reinforcement, shrinkage joints, and cast-in anchors. The Contractor shall ensure all reinforcement remains correctly positioned during grouting, with any displacement corrected at the block layer’s expense.

Cleanliness & Workmanship: Prior to grouting, all vertical cores shall be cleared of excess mortar, and the first course shall be thoroughly cleaned of sand and mortar droppings. Penetrations shall be formed during construction, avoiding block cutting where possible.

Observation Level: Construction observation shall be Type B as per NZS 4230.

8.4 CONNECTION & TOLERANCE REQUIREMENTS

Steel Connections: All bolts, washers, plates, and connectors shall be hot-dip galvanized. Washer sizes shall comply with NZS 3603:1992 (Clause 4.4), with minimum dimensions as follows:

- M8 bolts: 25 × 25 × 1.5 mm

- M12 bolts: 50 × 50 × 3 mm

- M20 bolts: 65 × 65 × 5 mm

Tolerances: Blockwork shall maintain ±5 mm vertical alignment, ±3 mm per 3 m horizontal alignment, and ±2 mm joint thickness variation. Non-compliant work shall be rectified by the Contractor at their own cost.

  - QUALITY ASSURANCE

The Contractor shall ensure all materials, workmanship, and construction methods meet the specified standards. Final approval shall be contingent upon Engineer verification of compliance with structural drawings, NZ Building Code, and contract documents.

  - REINFORCEMENT REQUIREMENTS

Lapping of Reinforcement

All reinforcing bar laps in masonry blockwork shall comply with the following minimum requirements:

| Bar Diameter (mm) | Grade 300E Minimum Lap Length (mm) | Grade 500E Minimum Lap Length (mm) |
| --- | --- | --- |
| 10 | 400 | 700 |
| 12 | 500 | 850 |
| 16 | 650 | 1150 |
| 20 | 800 | 1400 |

Note: Lapped bars shall be properly tied and positioned within the grouted core.

Concrete Cover Requirements

Minimum concrete cover to reinforcement shall be measured from the external face of the blockwork to the center of the reinforcing bar, complying with:

- NZS 4210:2001 "Masonry Construction: Materials and Workmanship"

- NZS 3101:2006 "Concrete Structures Standard"

The minimum nominal cover to all reinforcement shall be:

| EXPOSURE ZONE | MINMUM COVER (mm) |
| --- | --- |
| ZONE B and Interior conditions | 45 |
| ZONE C | 50 |
| ZONE D | 60 |

The Contractor shall verify all reinforcement placement maintains these minimum cover requirements prior to grouting. Any non-compliant work shall be rectified at the Contractor's expense.